SECTION 11 6823

### PART 1 GENERAL

  - RELATED WORK

Review Contract Documents for requirements that affect work of this section. Specification sections that directly relate to work of this section include, but are not limited to:

Section 11 6500 - 	Athletic & Recreational Equipment

Section 12 9300 - 	Site Furnishings

Section 31 2300 - 	Excavation and Fill

Section 33 3000 - 	Site Improvements

Section 32 1820 - 	Athletic & Sports Surfaces

Section 32 1826 - 	Synthetic Grass Surfacing

Section 32 9200 -	Turf and Grasses

1.02	SUBMITTALS

A. Product Data: Submit manufacturer’s product literature, technical specifications, and other data required to demonstrate compliance with specified requirements for all athletic equipment.

1.03	QUALITY ASSURANCE

A. Fabrication and installation of site improvements by experienced craftsmen with excellent record of performance on completed projects of comparable size, scope, and quality.

B. All materials, hardware and furnishings shall be new, first quality.

1.04	FIELD MEASUREMENTS

A. Contractor shall verify position and layout of rubber border. Verify dimensions by field measurements.

  - SYSTEM DESCRIPTION

- Multi-purpose Rubber Transition Ramp, .25” High (Front) / 1.50” High (Rear) x 6” Wide x 48" Long, shall be manufactured of 100% recycled rubber, Model # SEFLEXRR150.6

### PART 2 PRODUCTS

2.01 	FlexEdge® Rubber Transition Ramp

A. Base: Model #SEFLEXRR150.6 by:

SportsEdge®

259 Murdock Rd.

Troutman, NC 28166

P: 800-334-6057

info@sportsedge.com

www.sportsedge.com

B. COMPONENTS: Synthetic Turf Border System and Rubber Transition Ramp, shall be manufactured of 100% recycled rubber, and must meet the following criteria as the minimum standard:

1. Rubber L Curb:

a. .25” High (Front) / 1.50” High (Rear) x 6” Wide x 48" Long

b. Manufactured of 100% recycled rubber
c. Manufactured of 90% post-consumer content

2. Accessories:

- Regupol PU350 Low VOC Flooring Adhesive, 2.5 or 5 Gal., part # SEFLEXR25G OR SEFLEXR5G

- Betaseal Adhesive, 10.5 oz., part #SEFLEXBST

- UV Top Coat - 4 Liter, part #SEFLEXTC

### PART 3	EXECUTION

3.01 INSTALLATION OF EQUIPMENT

A. Install as recommended by the manufacturer, and as indicated on the drawings. The installing contractor shall ensure that the base is compacted and level to correct elevation.

END OF SECTION