SECTION 02880

### PART 1 GENERAL

  - RELATED WORK

Review Contract Documents for requirements that affect work of this section. Specification sections that directly relate to work of this section include, but are not limited to:

Section 02200 – Site Preparation

Section 02542 - Synthetic Grass & Turf

Section 02790 - Athletic & Recreational Surfaces

Section 02870 – Site Furnishings

Section 03300 – Cast In Place Concrete

Section 11480 - Athletic, Recreational and Therapeutic Equipment

1.02	SUBMITTALS

A. Product Data: Submit manufacturer’s product literature, technical specifications, and other data required to demonstrate compliance with specified requirements for all athletic equipment.

1.03	QUALITY ASSURANCE

A. Fabrication and installation of site improvements by experienced craftsmen with excellent record of performance on completed projects of comparable size, scope, and quality.

B. All materials, hardware and furnishings shall be new, first quality.

1.04	FIELD MEASUREMENTS

A. Contractor shall verify position and layout of all athletic field equipment. Verify dimensions by field measurements.

  - SYSTEM DESCRIPTION

A. Double Baseball Batting Cage System consisting of (2) 12’H x 14’W x 80’L cages with 8.625” diameter structural steel poles and STRYK-5388 rust inhibitive black coating. Cable System comprised of 3/16” galvanized steel aircraft cable. Net shall be 380 Denier HDPE Yarn, 3 strand, #42 twine.

### PART 2 PRODUCTS

2.01 	BASEBALL BATTING CAGE SYSTEM

A. Base: Model #SEDBBC-01 by:

SportsEdge

P.O. Box 837

259 Murdock Rd.

Troutman, NC 28166

P: 800-334-6057

info@sportsedge.com

www.sportsedge.com

B. COMPONENTS

SUMMARY: Double Baseball Batting Cage System shall consist of (2) 11’H x 14’W x 80’L cages meeting the following criteria as a minimum standard:

1. Structural Poles:

a. 8.625" O.D. welded high tensile strength steel, .25” wall thickness with welded cap and integral lifting ring to prevent moisture from entering at the top and to ease the installation process.

b. Corrosion dipped, and finished with STRYK 5388 to form a solid coating bonded with the substrate, preventing corrosion.

2. Cable and Tensioning System

a. All hardware including nuts, bolts, washers, are to be hot-dipped galvanized.

b. Cable shall be 3/16” galvanized aircraft cable. Cable tensioning must allow for adjustment of any given cable.

c. 5/8” x 36” Galvanized All-Thread (Threaded Rod)

d. Galvanized Threaded Thimble Eye Nuts

e. 5/8” x 14” Cast Oval eye Bolts

f. Galvanized Curved Washers

g. 5/8” Galvanized Nuts

h. 3/16” Cable Clamps

3. Netting

a. 1-7/8” x 1 7/8” Openings

b. Color: Black

c. Rope Diameter: 5/16” Twisted HDPE

d. YARN: 380 DENIER

e. Tensile:  280# breaking strength

f. Anti UV Active Content: .02%

4. Accessories:

- ½ gallon STRYK-5388 touch up coating

### PART 3 EXECUTION

3.01 INSTALLATION OF EQUIPMENT

A. All athletic equipment shall be installed as recommended by manufacturer, and as indicated on the drawings.

END OF SECTION